| SR NO. | NAME OF ITEM | Description | QUANTITY | UNIT | RATE | per | amount in rupees |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Excavation | Earthwork excavation in open cutting upto 5'-0" (1.5 m) 17.1 to depth for storm water channels, drains, sullage drains in 17.5 open areas, roads, streets, lanes, including under pinning of walls and shoring to protect existing works, shuttering and timbering the trenches, dressed to designed level and dimensions, trimming, removal of surface water from trenches, back filling and surplus excavated material disposed of and dressed within 50 ft. (15 m) lead:- | 935.516 | cft | 7,623.55 | 1000 cft | 7128.01925 |
| 2 | Concrete | Cement concrete plain including placing, 20.1 compacting, finishing and curing complete (including screening and washing of stone aggregate): | 915.34 | cft | 22977.75 | 100cft | 210324.5369 |
| 3 | Brickwork | Pacca brick work in foundation and plinth in:-(cement, sand mortar(1:4)) | 1065.228 | cft | 27027.5 | 100cft | 287904.4977 |
| 4 | paint exterior coat 2 | Providing and applying weather shield paint of approved quality on external surface of building including preparation of surface, application of primer complete in all respect: | 1520.21 | sft | 960.85 | 100sft | 14606.93779 |
| 5 | paint exterior coat 1 | Providing and applying weather shield paint of approved quality on external surface of building including preparation of surface, application of primer complete in all respect: | 1520.21 | sft | 1957.4 | 100sft | 29756.59054 |
| 6 | Brick work | Pacca brick work in ground floor:-i) cement, sand mortar:- | 1402.532 | cft | 23707.6 | 100cft | 332506.6764 |
| 7 | paint interior coat 2 | Providing and applying weather shield paint of approved quality on external surface of building including preparation of surface, application of primer complete in all respect: | 3249.21 | sft | 446.15 | 100sft | 1990.498225 |
| 8 | paint interior coat 1 | Providing and applying weather shield paint of approved quality on external surface of building including preparation of surface, application of primer complete in all respect: | 3249.21 | sft | 865.5 | 100sft | 28121.91255 |
| 9 | Wood work | First class teak wood wrought joinery in doors and windows, etc. paneled, paneled and glazed or fully to glazed and fixed in position, including chowkat, holdfast, tower bolt, chocks, rubber stop, cleats/G.I. clamps, & chords with hooks, nails, screws, etc. complete (excluding sliding bolt and lock):- | 119 | square ft | 3774 | sft | 449106 |
| 10 | Glasswork+Aluminium | Providing and fitting all types of glazed aluminum windows of anodized bronze color partly fixed and partly sliding using deluxe sections of approved manufacturer having frame size of 100 x 20 mm (4"x¾") and leaf frame sections of 50 x 20 mm (2"x¾"), all of 1.6mm thickness including 5 mm thick imported tinted glass with rubber gasket using approved standard latches, hardware c., as approved by the Engineer in-charge. | 115.83 | square ft | 477 | sft | 55250.91 |
| 11 | plaster | Cement plaster 1:4 upto 20' (6.00 m) height:- ½" (13 mm) thick | 4356.69 | sft | 2153.8 | 100 sft | 93834.38922 |
| 12 | Steel | Fabrication of mild steel reinforcement for cement concrete, including cutting, bending, laying in position, making joints and fastenings, including cost of binding wire and labor charges for binding of steel reinforcement (also includes removal of rust from bars):-) Deformed bars (Grade-40) | 4165.61 | lb. | 8090.45 | cwt=112lb | 300907.6734 |
| 13 | Marble | Providing and laying floor of mosaic marble chips tiles 1"(25 mm) thick of approved quality and shade, laid in white cement and pigment, over ¾"(20 mm) thick bedding of cement sand mortar 1:2, including finishing and polishing, complete | 903.072 | sft | 9,226.45 | 100sft | 83321.48654 |
| 14 | Tiles | Cement concrete tiles laid in 1:2 cement mortar, over 3/4" (20 mm) thick bed of cement mortar 1:2:-a:) 12"x12"x1" (300 x 300 x 25 mm) | 283.2 | sft | 4,971.35 | 100sft | 14078.8632 |
| 15 | D.P.C | Providing and laying damp proof course of cement concrete 1:2: 4(using cement, sand and shingle), including bitumen coating :- 1½" thick (40 mm) | 11.82666667 | sft | 5112.35 | sft | 60462.05933 |
| 16 | Backfilling | Earthwork excavation in open cutting upto 5'-0" (1.5 m) 17.1 to depth for storm water channels, drains, sullage drains in 17.5 open areas, roads, streets, lanes, including under pinning of walls and shoring to protect existing works, shuttering and timbering the trenches, dressed to designed level and dimensions, trimming, removal of surface water from trenches, back filling and surplus excavated material disposed of and dressed within 50 ft. (15 m) lead:- | 149.712 | cft | 7,623.55 | 1000cft | 1141.336918 |
| 17 | skirting tiles | Tile skirting laid in 1:2 cement mortar, over ¾"(20 mm) thick cement mortar, 1:2 including cement washing and filling joints complete:- | 78.5614 | sft | 12,287.25 | 100sft | 9653.035622 |
| 18 | Pacca Brick Work in Roof (cement, sand mortar(1:4)) | Pacca Brick Work in Roof (cement, sand mortar(1:4)) Add extra labor on item No. 5 for brickwork in:- | 36.6273 | cft | 24668.05 | 100cft | 9035.240678 |
| 19 | Main gate | Making and fixing steel grated door with 1/16" thick (1.5mm) sheeting, including angle iron frame 2"x2"x3/8" (50x50x10 mm) and ¾" (20 mm) square bars 4" (100 mm) center to center, with locking arrangement. | 48 | sft | 1251.95 | per sft | 60093.6 |
| 20 | Two coats of bitumen | Two coats of bitumen laid hot using 34 lbs. per %Sft, or 1.72 Kg per square meter over roof and blinded with sand at one Cft. per %Sft. (0.003 cu.m per sq.m) | 903.05597 | sft | 2137.7 | 100sft | 19304.62747 |
| 21 | Two layers of Polythen sheet | Providing and Laying Insulation material of Extruded Polystyrene XPS in Rigid Insulation / Foam Board on roof or walls, Density 32-38Kg/M, compressive strength 250-400 kpa, R-value 5 per inch thickness and water absorption (1% by volume, cell structure clored cell) i/c cutting and placing in position. complete in all respect. | 903.05597 | sft | 10865.9 | 100sft | 98125.15864 |
| 22 | Well compacted earth | Earth flooring, consisting of 6"(150 mm) thick consolidated layer of moistened earth, including ramming. | 2108.78 | cft | 3516.5 | 1000cft | 7415.52487 |
| 23 | 6" Sand filling | Supplying and filling sand under floor; or plugging in wells | 490.53 | cft | 2103.15 | 100Cft | 10316.5817 |

Total	2184386.157